Mapping Drinking-Water Risk in Turkana to Guide Intervention

Fred Omengo1,2,4, Sospecter Njeru3, Julien Ayrolesx,2,3, Elizabeth Niespolo1, *

1- Department of Geosciences, Princeton University, Princeton, NJ, USA

2- Turkana Basin Institute, P.O. Box 24467, 00502, Nairobi, Kenya

3- Center for Community Driven Research, Kenya Medical Research Institute, Kirinyaga, Kenya.

4- Wildlife Research and Training Institute, P.O. Box 842-20117, Naivasha, Kenya.

X - Department of Integrative Biology, University of California, Berkeley, Berkeley, CA, USA

Keywords: Turkana County, potable Groundwater, Boreholes, Fluoride, geogenic contamination

Correspondence: fo5058@princeton.edu

ABSTRACT

Access to safe drinking water remains a critical challenge in arid regions, where communities rely almost exclusively on natural groundwater. Despite the heavy dependence on ground water resources in majority of arid areas of Africa, their chemical and microbial quality remains poorly understood, potentially posing risks to public health. We analyzed 152 water sources from Turkana County, Kenya. The sources included deep and shallow boreholes, open hand-dug wells, and natural springs, which represent the main potable water sources in the region.

Forty-eight physical and chemical parameters were measured and compared to existing guidelines; World Health Organization and KS 459-1:2007 drinking water guidelines. Out of the 32 parameters with existing guideline values, 21 parameters exceeded minimum contaminant levels (MCL) at up to 42% of the sites. Elevated TDS (range: 0.2 -11 mg/L) and widespread contamination from both geogenic sources and microbial activity were prevalent in majority of the watering points. Salinity was driven by sodium, calcium, potassium and magnesium salts, with associated carbonate and sulphate compounds similarly showing concentrations above guideline values. Fluoride (MCL = 1.5 mg/L) was among the most pervasive chemical hazard, with nearly half of all deep and shallow boreholes exceeding existing guideline values (maximum F = 26 mg/L). Manganese and aluminum salts were found in excess primarily in deep borehole water sources, whereas shallow open wells were characterized by heavy microbial contamination. Our findings indicate that while deeper mechanized boreholes provide reliable yields, they frequently breach chemical guidelines, and while shallow wells dug on dry riverbeds have relatively lower dissolved loads, they pose acute microbial risks.  The findings from Turkana illustrate how climate-driven aridification and changing lifestyle has intensified risks of chronic hydrotoxic exposure with potentially perilous health implications.

INTRODUCTION

Access to safe and reliable water remains a persistent global challenge, particularly in remote and arid regions. Over time, many indigenous communities have developed complex systems and adaptive strategies for accessing, managing, and safeguarding these scarce and seasonal resources, often relying on intricate ecological and cultural knowledge passed across generations (Sharifian et al. 2022). In the recent past, accelerating climate change and shifting land-use are disrupting these longstanding systems, undermining traditional forecasting methods and exposing communities to unprecedented environmental stress.

In arid and semi-arid regions like northern Kenya, escalating water scarcity and demographic pressures (such as increasing populations) are forcing a transition from traditional surface water sources from rivers and shallow wells to mechanized, deeper groundwater systems. This shift, centered on a growing reliance on deep boreholes for domestic and livestock water, reflects a broader trend across climate-vulnerable parts of Sub-Saharan Africa (Healy et al. 2022). This transition often delivers more reliable access to water in drought-prone environments, where shallow sources often fail seasonally. Accessing deeper ground water introduces a complex trade-off between reliability and health hazards related to either microbial or chemical loads in water resources. The deep aquifers typically provide pathogen-free water through natural filtration, but this geological isolation also drives the accumulation of geogenic contaminants through water-rock interactions. Despite this, the majority of water development initiatives tend to define safe drinking water by its microbial quality, even though this narrow focus can mask serious health risks posed by water chemistry. Mineral content alongside taste ultimately determines a water source's usability and its effect on long-term health (Nowicki et al 2023). Water that is reliably available and free of microbial contamination may still contribute to chronic illness through chemical exposure, or be rejected by communities due to poor taste, creating new health burdens and risks that offset the intended benefits.

Chemical characterization of Turkana County's water resources remains geographically biased and incomplete. Most water development projects prioritize yield over water quality, and existing assessments concentrate around major towns and rarely include comprehensive element analyses. Critical contaminants such as arsenic, fluoride, and lead are usually underreported, even in relatively well-studied catchments like the Turkwel River basin in Lodwar (Fig.  1). Beyond a lack of data, existing studies suffer from poor integration across disciplines, as available datasets employ narrow parameters and fail to connect hydrochemical findings with underlying geology, for example, or public health outcomes. Although unsafe water consumption has been linked to health complications in the region, studies that systematically pair chemical monitoring with epidemiological surveillance are notably absent. This disconnect significantly limits the ability to quantify population-level exposure risks, identify vulnerable subgroups, and design evidence-based interventions tailored to Turkana's complex hydrogeological setting.

As drought frequency increases across sub-Saharan Africa, deep borehole development has accelerated as a key response to increasing climatic variability and water stress (Cuthbert et al 2019; Calow, et al 2010). This rapid expansion has outpaced water quality assessment capacity, and most projects limit chemical analyses to initial pump testing, if conducted at all (Cronin et al., 2006; Lapworth et al., 2017). Even when initial testing shows concentrations exceeding drinking water standards, remedial measures such as decommission contaminated sources, improvement to treatment infrastructure, or public health advisories are rarely implemented (Lapworth et al., 2017). Groundwater is a vital resource for rural populations across sub-Saharan Africa, but its potability is fundamentally controlled by local geology. While interactions between water and rock can enrich groundwater with beneficial minerals, these same processes can also contaminate it with harmful excesses of certain elements, rendering it unsafe for consumption (Edmunds & Smedley, 1996; Davies & MacDonald, 2002).

Recent evidence underscores the scale of the challenge: Nowicki et al. (2023) reported that 96% of groundwater samples from selected regions of Kenya and Ethiopia exceeded at least one WHO guideline for inorganic drinking water quality, originating from the underlying geology, saline intrusion, and anthropogenic contamination from waste disposal and land-use change. Turkana County exemplifies these challenges. Located within the East African Rift System, it sits in a geologically complex landscape shaped by active tectonism, faulting, and heterogeneous hydrogeological settings. Four main aquifer subsystems have been identified: shallow alluvial, intermediate, deep, and the Turkana Grit, a late Cretaceous fluvial-dominated, coarse-grained

Sandstone, with recharge driven largely by precipitation and supplemented by subsurface flows along riverbeds and ephemeral water courses (laggas) (Tanui et al., 2023; Kuria, 2013; Makhoha & Kweyu, 2024).

This complex hydrogeology is a primary driver of water contamination, giving rise to distinct chemical signatures across Kenya. For example, high iron concentrations are commonly associated with metamorphic aquifers in the Mozambique Mobile Belt, while lead has been found in coastal sedimentary aquifers (Gavera et al., 2024). Within the Rift Valley system, fluoride has been a dominant concern. Fluoride arises predominantly from geogenic sources, particularly in sodium-bicarbonate (Na-HCO₃) type waters characteristic of the rift valley region (Rusiniak et al., 2021).

Driven by government agencies, non-governmental organizations, and private stakeholders promoting centralized water provision, communities in Turkana have increasingly shifted from traditional river water and sand filtered hand-dug wells to mechanically drilled boreholes. This transition over recent years has resulted in over 1,200 boreholes and shallow wells across the county (WRA, 2018). In the majority of these water infrastructures, local administrative leaders, including chiefs and water resource elders have been entrusted with overseeing their operation. However, this growing dependence is met with significant and underappreciated safety concerns, as a limited number of studies (Rusiniak et al., 2021) indicate that these sources exceed WHO guidelines for fluoride, microbial contamination, and other parameters.

Although Turkana populations have demonstrated remarkable resilience to climatic and hydrological variability over centuries (Wright et al., 2015, A. J. Lea et al., 2025), emerging chemical threats from groundwater pose unprecedented public health risks. These risks are compounded by intensifying droughts, resource competition, and climate-driven water insecurity (Roba et al., 2025). Chronic exposure to specific contaminants has been linked to severe health outcomes; for example, elevated salinity is associated with hypertension and impaired renal function (Rosinger et al., 2021). Excessive concentrations of other trace elements are linked to a wider range of chronic illnesses, including immune function, cardiovascular disease, and increased risks of certain cancers and neurological disorders (Edmunds & Smedley, 1996; Khan et al., 2014; Naser et al., 2019; Ebi & Bowen, 2016).

Despite comprehensive water resource legislation in Kenya, borehole development in Turkana often proceeds without adequate geophysical surveys or water quality assessments. Critical parameters are poorly characterized or underreported, despite well-known geogenic contamination risks in Rift valley settings (Kuria, 2013). Water safety monitoring remains reactive and fragmented, undermining sustainable groundwater development.

This study aims to improve our current understanding of groundwater chemistry and potability in Turkana County and provide data for investment, water resource management, and county leadership. We present comprehensive chemical analyses of 48 physical-chemical parameters in 152 representative water sources across the four most common water access points found in Turkana County: deep solar-powered boreholes (DBH Solar), shallow hand-pumped boreholes (SBH pump), open hand-dug wells (OHD wells), and natural springs (Springs). We overlay our results on existing major lithological types that would potentially drive the measured chemistry across these source types and evaluate the potential health implications of elevated concentrations against World Health Organization (WHO) and Kenya Bureau of Standards guideline for drinking water (KS 459-1:2007). We map these results on the landscape to develop a hydrochemical baseline across diverse water source types to identify priority intervention sites to enable health risk mapping and guide informed intervention by local institutions.

Fig 1: General map Of Turkana Lithology and sampling points

MATERIALS AND METHODS

#### Study Area

Turkana County is Kenya's largest county by land area at about 77,597 km². The county is located in the northwestern area of Kenya, and borders Uganda to the west, South Sudan and Ethiopia to the north, and the Kenyan counties of West Pokot, Baringo, and Marsabit to the south and east. The region has an arid to semi-arid climate with high annual average temperatures (20°C to 37°C) and low, bimodal rainfall averaging less than 250 mm/year. Evapotranspiration rates are high, reaching up to 2800 mm/year (NDMA, 2016). Because of the high evapotranspiration rates, and the fact that the lake has no outlet, lake levels in Lake Turkana can vary widely with changes in precipitation (Bloszies et al., 2015) and shallow water resources are likely to be affected similarly. Rainfall has historically been bimodal, with a primary wet season occurring from March to May and a secondary, less intense season from October to December (Opiyo, 2014). However, in recent years these patterns have become less predictable, leading to less accurate water resource forecasting. Elevation across the study area varies from about slightly above1000 m in the western upland zones to 400 m above sea level near Lake Turkana, with the terrain sloping eastward toward the lake basin (Fig 1).

Turkana County is administratively divided into six sub-counties: Turkana North, Turkana West, Turkana Central, Loima, Turkana East, and Turkana South. Settlement patterns across the county are largely influenced by water availability and basic infrastructure, given the region's arid conditions. Lake Turkana constitutes the lowest point in the county, receiving surface runoff from multiple drainage basins. The Omo River, originating in Ethiopia, flows directly into the Lake from the north, while two other major rivers Turkwel and Kerio Rivers drain into the lake from the south/southwestern side. Additional seasonal rivers and ephemeral streams (laggas) traverse the county, most of which are fed by local precipitation and are critical for both surface recharge and temporary storage.

The county hosts several significant groundwater resources, in the recent past, four major aquifers have recently been identified: Lotikipi, Gatome-Nakalale, Kachoda, and the Lodwar–Lokichar (Napuu) aquifer system. The latter extends through much of central and southern Turkana. These aquifers have been described as relatively deep, ranging from 80 m to over 700 m in depth, especially in sections of the Lodwar–Lokichar system (UNESCO, 2013). In some areas, smaller perched aquifers occur at shallower depths (30-70 m) and generally exhibit better water quality (UNESCO, 2013). However, most of these aquifers remain poorly characterized and underutilized (UNESCO, 2013).  Despite the existence of these deep aquifers, numerous seasonal laggas provide shallow groundwater storage within sandy riverbeds, accessed primarily through traditional hand dug wells (Fig. 2). These sources represent the traditional approach to groundwater access; they are vital during dry seasons and are widely distributed across the landscape.

Fig 2: (a) Hand dug well (OHD well), (b) shallow borehole with hand pump (SBH Pump), (c) Deep borehole with solar ( DBH solar) and (d) Spring water source (≈ depths OHD well <5m, SBH Pump < 20m, DBH solar > 50m)

Turkana county exhibits a tectonically diverse lithology shaped by active rifting processes, volcanic activity, extensive sedimentary basin development, and associated geothermal systems (Tiercelin & Lezzar, 2002; Dunkley et al., 1993). The oldest rocks in the region belong to the Precambrian crystalline basement complex, which comprises metamorphosed sedimentary and volcanic sequences, along with mafic and ultramafic intrusions. These rocks have undergone multiple episodes of regional metamorphism, compressional deformation, and orogenic intrusive events (Walsh & Dodson, 1969). Exposures of the crystalline basement are most prominent in the eastern and southern margins of Turkana, particularly near Lokiutang and Lodwar. Overlying the basement complex are extensive deposits of Cretaceous to Quaternary sediments, including the Late Cretaceous Lapur Sandstones (colloquially called the “Turkana Grits’; Agyemang et al., 2019), and fluvial-lacustrine facies of the Nachakui, Shungura, and Koobi Fora formations, which are of international significance due to their well-preserved hominid fossil assemblages (e.g., Walker & Leakey, 1993; McDougall et. al, 2012; Feibel, 2011; Leakey et al., 2011; Grine et al., 2026; Mongle et al., 2025). These are further overlain by recent Quaternary alluvial and lacustrine deposits. Shallow aquifers typically occur within alluvial deposits along river valleys, while deeper aquifers, such as the Lotikipi Basin Aquifer, are situated within the sedimentary basins

Methods

Water samples were collected from existing water sources across Turkana County, that included deep boreholes (>25 m; DBH Solar), most of which were equipped with solar-powered pumps; shallow boreholes (<25 m; SBH pump) typically fitted with mechanical hand pumps; open hand-dug wells (average 0.5 - 2 m; OHD wells) located primarily within dry riverbeds and ephemeral streams (laggas) and natural springs with free-flowing discharge. The identification and selection of sampling points was carried out in collaboration with local administrative leaders, including chiefs and water resource elders, within each ward visited.

Sampling:

At each site, two sets of water samples were collected for analysis: (i) a sample for commercial laboratory in Kenya (Cropnuts Laboratory, Nairobi), a Kenya National Accreditation Service (KENAS)-accredited facility, and (ii) a second sample for analysis at the Department of Geosciences of Princeton University (New Jersey, USA). For the analyses conducted at Cropnuts Laboratory, water was collected in pre-sterilized 1-liter high-density polyethylene (HDPE) bottles, filled directly after purging the wells by allowing the water to flow gently until overflowing, then capped tightly with minimal headspace. Samples were stored in a mobile refrigerator at sub-zero temperatures and transported by air to Nairobi within 3-4 days for analysis. For the U.S. analyses, water was collected in 60 mL sterile HDPE bottles, each sample was filtered onsite through 0.22 µm Millipore filters using a sterile syringe system and immediately preserved with 200 µL of ultrapure dilute nitric acid (2–3% v/v) to stabilize dissolved elements and prevent metal precipitation or microbial degradation. Samples were stored at ambient temperature and shipped to the laboratory at Princeton University, where they were analyzed approximately 40 days after collection.

Prior to sample collection, all boreholes, including both motorized and hand-pumped systems, were purged for approximately 10 minutes to ensure a representative sample was collected. Following this, water samples was drawn into a clean bucket and in situ parameters that included pH, conductivity, temperature, and salinity, were measured using a pre calibrated YSI Professional Plus multiparameter meter (Model P2030 2030-4P) and a VWR pH1100H pHenomenal® pH/mV/°C meter. The YSI multiparameter meter converts temperature-compensated specific conductance ( 25  C) by a conversion factor (k), in our case was 0.55.

Sample Chemistry:

The following elements were analyzed in samples at Cropnuts Laboratory (Kenya): P, Ca, Mg, K, Na, Mn, Fe, Cu, Mo, B, Zn, S, and Si, and heavy metals, using an Inductively Coupled Plasma Optical Emission Spectroscopy (ICP-OES) instrument (resolution ~1 ppm). Ammonium, nitrate-nitrogen, chloride, bicarbonate, and fluoride were determined via colorimetric methods, using specific reagents and measuring absorbance at designated wavelengths and reported in ppm. Total dissolved solids (TDS) and total suspended solids (TSS) were determined gravimetrically by filtering a known volume of water through a standard glass fiber filter, followed by drying at 103°C to constant weight.

All samples were measured for major and trace elements at Princeton University with an Agilent 8900 ICP-QQQ mass spectrometer (Agilent Technologies, Japan) operating in tandem mass spectrometry mode. The instrument was fitted with an ESI SC-4 DX autosampler, x-lenses, and the standard sample introduction kit (MicroMist concentric glass nebulizer, Peltier-cooled quartz spray chamber at 2 °C, quartz torch with 2.5 mm injector). Depending on the sample concentration, 100 – 200 µL aliquot of each sample was transferred to pre-cleaned acid-leached polypropylene ‘rocket’ vials, diluted by 2% v/v HNO₃ to 2 mL and spiked with 50µL of Indium as a matrix-matching stabilizer. Highly concentrated samples were diluted 1:1000 with the same acid matrix. Daily instrument performance tuning used a multi-element solution (Li, Co, Y, Ce, Tl; 10 µg/L) to achieve an oxide ratio (CeO⁺/Ce⁺) < 1 %, doubly charged ratio (Ce²⁺/Ce⁺) < 2 % and sensitivity > 1.0 × 10⁵ counts s⁻¹ for ¹⁴¹Pr.

Two standards were used for calibration, National Institute of Standards and Technology 1643f (Trace Elements in Water) and multi elemental calibration solution ICP-MS-68A prepared from single-element ICP stock solution and gravimetrically diluted in 2 % ultrapure HNO₃. Calibration curves were generated by weighted (1/x) linear regression with coefficients of determination R² ≥ 0.999 for all monitored elements. Indium was used as an internal standard in all measurements. For quality control, procedural blanks (2 % HNO₃) were analyzed every ten samples. Raw counts were corrected for blank and internal standard response and converted to concentrations using Agilent MassHunter 5.1 and expressed in µg/L.

Data Analysis:

After full data treatment, figures were prepared with the R software (R version 4.5.1, 2025 The R Foundation for Statistical Computing) using the function “boxplot” and the package “ggplot2”. Our results are reported and compared to WHO guidelines for drinking water (WHO, 2022) and additionally with the Kenya Bureau of Standards guideline for drinking water KS 459-1:2007 (KEBS, 2007), hereby abbreviated as (KS).

RESULTS

Sample Distribution

A total of 152 water points were sampled across Turkana County, representing four primary source types: deep solar-powered boreholes, (DBH Solar, n=90), shallow hand-pumped boreholes, (SBH pump, n=38), open hand-dug wells, (OHD wells, n=21) and springs (n=3). Sampling covered 23 of out of the 30 wards in Turkana County. This distribution was limited by safety and security constraints, preventing access to Letea, Lobei, Kaputir, Katilla, Kochidin, Kapedo and Lobokat wards. Subsampling maximized spatial representation in areas with dense water points, targeting varied locations along different drainage systems. For major and trace elements, results from both Cropnuts laboratory and Princeton Laboratory were comparable, therefore only data from Princeton laboratory has been reported here due to higher instrument detection limit (to ppt for some elements) as opposed to ppm from the Cropnuts laboratory. Results of Fluoride, Chloride and compounds NH4, HCO3, SO4, CaCo3, NO3N and SiO2 are based on analysis from Cropnuts laboratory in Kenya. It is worthy to note that majority of parameters reported here recorded extreme variability across the landscape and water source types, for a true threshold, results have been reported in median to address significant outliers.

General Physicochemical Parameters

Conductivity and corresponding Total Dissolved Solids (TDS) values were elevated in both SBH pump and DBH Solar with a highly sporadic distribution within the county, highest concentration (> 2300 µS/cm or >1200 mg/L TDS) being in Lokichar, Kerio, Katilu and Lokitaung wards. Considering that majority of Turkana County is covered by quaternary sediments with alternating Precambrian basement rocks (Fig 2 & Fig 3), we could not directly associate the elevated conductivity and TDS with either of the lithologies. Across all water sources conductivity ranged from 304 µS/cm to 21,000 µS/cm (167 – 11,000 mg/L TDS), with a median of 1304 µS/cm ( 717 mg/L TDS). OHD wells recorded the lowest median TDS 375 mg/L, while the SBH pump and DBH Solar recorded medians of 755 and 750 mg/L respectively. (Fig 3). The three springs samples had a median TDS of 1038 mg/L.

Carbonate hardness (CaCO₃) exhibited similar spatial patterns, with TDS where elevated concentrations across were recorded in scattered parts of the county. While boreholes in the Kerio region showed elevated TDS and pH, they did not exhibit correspondingly high carbonate hardness. High CaCO₃ levels were recorded around Katilu, Lokiriama, and Lokichogio wards. By water source type, DBH pump showed the highest CaCO₃ hardness (median 202 mg/L) closely similar to the SBH pump with a median of 183 mg/L. OHD wells had the lowest CaCO₃ hardness with a median of 118 mg/L.

pH values were generally alkaline on the western side near the lake, particularly in the southern western areas of Kerio, Kang’atotha and Kalokol wards, highest pH was observed in a SBH pump borehole in Kerio Ward (pH 9.2) (Fig 3). Average pH across the landscape was near neutral (median pH = 7.59), with a median pH of 7.76 for the OHD wells, 7.4, and 7.6 SBH pump and DBH solar respectively. Springs recorded an average median of pH 8.13, indicating generally neutral to slightly basic conditions.

Fig 3: Concentration of TDs, Fluoride and pH in the landscape, Boxplot dotted line represents WHO and KS guideline thresholds

Essential Elements and Ions Concentrations with potential toxicity

A number of major and trace elements that are considered essential to human body were measured across the Turkana landscape. While these elements are critical for physiological function and human health, elevated concentrations of certain elements can pose serious health risks.

Sodium (Na)

The median sodium (Na) concentration across all water sources was 101 mg/L, with a lowest median of 58 mg/L found in OHD wells, while SBH pump and DBH solar reported medians of 127 and 106 mg/L, respectively. Median concentration of springs was 139 mg/L. About 24% of all measured water sources categories exceeded the 200 mg/L safe limits as defined by Kenyan Standard KS 459-1:2007 (KS). Spatially, elevated concentrations were common in Lokichar ward, while 3 boreholes in Kerio had concentrations higher than 700mg/L. A number of boreholes in Kibish and Turkwell wards had elevated (> 500 mg/L) concentrations. In both cases the lithology was dominated by quaternary sediments and Precambrian rocks.

Potassium (K)

Median concentration of potassium across water sources in the landscape was 4 mg/L. with a range of concentrating between 0.7 – 31 mg/L. Elevated levels above 20 mg/L were recorded in a few DBH solar in Lokichar, and to a lesser extent in Katilu and Turkwel wards. Although many individual sites had sporadic high concentrations, there was no significant variability  across water source types or within the different lithologies.

Magnesium (Mg)

Magnesium was among the most abundant elements recorded across all water sources. A number of sites had elevated concentrations above the 50 mg/L WHO limit for taste and a few exceeding the 100 mg/L safety threshold per Kenyan drinking water standards. The overall median concentration across all water types was 13 mg/L; OHD wells recorded the lowest median concentration of 5.1 mg/L, while SBH pump and DBH Solar boreholes reported medians of 18.3 and 15.8 mg/L, respectively.  Concentrations above 50 mg/L were recorded in Lokitaung, Turkwell, Lokichar and Kalobeyei wards. A few SBH pump in Lokitaung had concentrations above 75 mg/L.

Figure 4: Concentration of major elements, Boxplot dotted line represents WHO and KS guideline thresholds

Trace Essential Elements of Health Concern

Iron (Fe)

Median Iron concentrations recorded across the landscape was 0.36 mg/L, 0.3 mg/L, 0.5 mg/L, for OHD wells, SBH pump and springs respectively which exceeded the threshold of safe limit of 0.3 mg/L as per KS standards. The median for DBH solar was 0.16 mg/L which is within acceptable limit, however, around 28% if these DBH solar boreholes exceeded the KS standard threshold of 0.3 mg/L. Elevated (above 0.5 mg/L) of Iron concentrations were common in Lokichogio, Lokichar and Turkwel wards.

Manganese (Mn)

Eight percent of all water sources exceeded WHO and KS Manganese threshold of 100 µg/L. Lower concentrations were observed in the other sources: median 7.7 µg/L in SBH pump, 1.7 µg/L in DBH solar, and 18.35 µg/L in spring water. Concentrations in OHD wells were highest reaching up to 1200 µg/L, with a relatively low median of 16 µg/L. About 25 % of the OHD wells (5 of 20) had their concentrations exceeding thresholds as compared to 10% (4 of 37) and about 3% (2 of 76) for the SBH pump and DBH solar respectively. Elevated concentrations were widespread in almost all the wards that were sampled and there was no prevalence in a particular lithology.

Chromium (Cr)

The majority of samples across all the water sources remained well below the WHO threshold of 50 µg/L. Median concentrations were 2.33 µg/L  for SBH pump, 6.0 µg/L  for OHD wells, 2.89 µg/L  for springs and  1.0 µg/L DBH solar where 2 boreholes in Lodwar township and Kalokol wards exceeded both the WHO and KS standards.

Nickel (Ni)

All measured concentrations were within WHO threshold of 70 µg/L. Median values were 1.11 µg/L in OHD wells, 0.74 µg/L for SBH pump, 0.36 µg/L DBH solar and 0.86 µg/L for springs.

Fig 5: Concentration of minor elements, Boxplot dotted line represents WHO and KS guideline thresholds

Anions of Physiological and Environmental Significance

Sulfate (SO₄²⁻)

WHO recommends a limit of 250 mg/L of sulfate in drinking water whereas the KS recommends a threshold of 400 mg/L. Concentrations in Turkana County varied widely but were generally within the thresholds. At a few isolated SBH pump boreholes(n=3) in Kerio and Kalokol wards concentrations were above 1000 mg/L, with maxima reaching 2,500 mg/L, 10 times the WHO guideline. Median concentration across water sources was however relatively low at 41.4 mg/L, with the SBH pump, and DBH solar recording the highest concentrations median of 54.1 and 43 mg/L respectively, and the OHD wells and springs recording relatively lower median concentrations of 25.1 and 36.5 mg/L respectively.

Nitrate (NO₃⁻)

The WHO threshold for NO₃⁻ in drinking water is of 45 mg/L, maximum concentrations recorded were up to 390 mg/L in Turkwel Ward, additionally values exceeding 150 mg/L in a number of boreholes were recorded in Loima and Lokitaung ward. In total 53% of watering points exceeded the WHO threshold (35 out of 66). Average concentration across water sources remained within safe limits median 10.6 mg/L, with median concentrations of 16.2, 12.8, 7.6, and 0.5 mg/L for DBH Solar, OHD wells, SBH pump and springs respectively.

Non-essential elements of major health concerns

Fluoride (F⁻), lead (Pb), aluminum (Al), mercury (Hg), cadmium (Cd), arsenic (As), and boron (B), are known to pose potential health risks in drinking water when present above acceptable limits. Although naturally occurring, their toxicity depends on chemical speciation, exposure duration, and population vulnerability, particularly among children and pregnant women.

Fluoride (F⁻)

Among the non-essential elements, Fluoride was the most prevalent contaminant with a considerable number of sites exceeding both WHO  and KS guideline of 1.5 mg/L. Maximum recorded concentrations reached 26 mg/L at Lokamarinyang Spring (Kibish) and two boreholes in Kerio and Turkwell wards, both exceeding 10 mg/L. Median fluoride concentrations by source type was 0.3 mg/L OHD wells and 3 mg/L spring water. Prevalence of fluoride was in the SBH pump and the DBH Solar where both had median concentrations of 1.4 mg/L and up to 50% of sites (35 of 66) measured exceeded threshold. Two of the three natural springs had fluoride above limit while one had concentrations within acceptable limit. Spatially, elevated concentrations were prevalent in Kerio, Katilu and Lokichogio wards, however in almost all the wards there were water sources exceeding the 1.5 mg/L threshold.

Lead (Pb)

Relatively low concentrations of lead were recorded across the landscape, majority were below the WHO and KS guideline of 10 µg/L. Five noticeable boreholes (SBH pump) located in Kerio, Kang’atotha, Turkwell, and Kibish recorded concentrations between 7 and 12 µg/L. These elevated concentrations were prevalent in the older SBH pump boreholes, suggesting that aging infrastructure may be the source of contamination. Median concentrations 0.7 µg/L, 0.9 µg/L, 0.5 µg/L, 0.7 µg/L for OHD wells, SBH pump, DBH solar and springs respectively.

Arsenic (As)

Most sites had concentrations below the WHO threshold of 10 µg/L. Afew exemptions included extremely high concentrations measured at Katiko SBH pump (Lake zone ward) that recorded 42 µg/L, two boreholes in Kalokol and Lokiriama wards and Lokamarinyang spring in Kibish ward that had levels exceeding 10 µg/L threshold. Average arsenic levels in all water sources was at a median of 0.86 µg/L with highest concentrations in springs at a median of 4.1 µg/L.

Cadmium (Cd)

The highest recorded cadmium concentration was 1.7 µg/L, with a majority of sites having a concentration less than 0.1 µg/L well below the WHO and KS guideline of 3 µg/L.

Aluminum (Al)

Aluminum is not considered essential to human health, and its toxicological profile remains uncertain at low levels. The WHO and KS guideline of 200 µg/L is based primarily on aesthetic considerations (taste, turbidity, color) and treatment efficiency, rather than direct toxicity.  Based on our data about 41% of all water sources measured exceeded this limit (57 out of 139). The median concentration across all water bodies was however low at 20.4 µg/L. Springs and OHD wells recorded the highest aluminum levels medians of 893 µg/L, 670 µg/L respectively. DBH solar and SBH pump had lower median concentrations of 6.9 µg/L and 48.1 µg/L, respectively. Elevated (above 250 µg/L) aluminum most common in in Lokichogio, Turkwell, Lokichar and Lokitaung wards.

Fig 6: Concentration of minor elements of health significance, Boxplot dotted line represents WHO and KS guideline thresholds

Conditionally Essential and Non-essential Elements, with non-desirable effects and imposed limits

Additional set of elements categorized as conditionally essential were analysed as chloride, calcium , zinc, phosphorus, copper, barium, cobalt, vanadium and beryllium . Median concentrations across all water sources of zinc, copper, barium, and cobalt were 15.6 µg/L, 4.9 µg/L, 65.5 µg/L and 0.2 µg/L respectively which are within WHO and KS recommended thresholds. All the elements had minimal spatial variability.

In contrast, calcium, chloride, vanadium, and phosphorus displayed notably elevated concentrations in a number of water sources. Three DBH solar in Kaatapakin, Ngimuriai had chloride concentrations above 2000 mg/L, overall, the median chloride concentration across water types was 53.8 mg/L with the SBH pump having on average a higher median chloride concentration of 71.2 mg/L. Median concentrations of 28.1 mg/L, 21.4 µg/L, and 179.1 µg/L were recorded for  calcium, vanadium and phosphorous.

Additional elements

A set of additional trace elements including boron (B), sulfur (S), uranium (U), strontium (Sr), rubidium (Rb), lithium (Li), and compounds like nitrite (NO₂⁻), phosphate (PO₄³⁻), and ammonium (NH₄⁺), were analyzed due to their environmental significance. Although generally not acutely toxic at low concentrations, their presence provides insight into underlying hydrogeochemical processes, aquifer vulnerability, and potential health risks under chronic exposure.

Boron (B)

Median boron concentrations across all water sources were low at 0.17 mg/L, compared to a WHO and KS guideline for drinking water of 2.4 mg/L. At least two DBH solar in Kerio Ward and one in Kalokol Ward and Lokamarinyang spring exceeded the 2.4 mg/L threshold.

Uranium (U)

Elevated levels of uranium above the WHO and KS limit of 30 µg/L were observed at 5 DBH sites and 1 SBH pump site. Highest concentration (150 µg/L) was recorded in Loochwa Borehole (Lokichar Ward), with the rest of the exceedances from Kerio, Kalokol, and Turkwel wards. Median uranium concentrations were 0.8 µg/L for OHD wells and springs, 2.2 µg/L and 1.7 µg/L for SBH pump DBH solar respectively.

Strontium (Sr)

No WHO or KS guideline exists for strontium, but concentrations exceeding 4 mg/L have been linked to adverse effects on bone development, in addition, the element is considered an important geochemistry tracer.  Concentrations higher that 4 mg/L was recorded in one DBH solar in Lokiriama. Median concentration across all the water sources was relatively low at 0.5 mg/L.

Sulfur

Sulfur concentrations remained below the WHO aesthetic guideline of 250 mg/L across all sources with a median concertation of 13.3 mg/L across all water source types. No distinct spatial pattern or significant variability was observed between water source types or the general landscape.

Rubidium (Rb) a trace element not considered a significant concern in drinking water. Element concentrations were generally low, with a maximum record of 300 µg/L and a median of 4 µg/L, without discernible pattern across water source types or within the landscape.

Fig 7: Concentration of minor elements of health significance, Boxplot dotted line represents WHO and KS guideline thresholds

### Microbial Contamination in High-Risk Shallow Wells

While the primary focus of this study was on hydrochemical risks, we conducted a targeted microbial assessment on OHD wells (n=10), this being the water source considered most likely to be impacted by biological contamination. Our analysis was intended to characterize microbial risks in the water safety trade-off. The limited sample size was dictated by severe logistical constraints, as the 24-hour maximum holding time for reliable microbial analysis required samples to be flown from remote sites to a laboratory in Nairobi.

All ten samples exhibited extremely high bacterial loads, with Total Viable Counts (TVC) exceeding 300,000 cfu/mL at both 22°C and 37°C temperatures, a clear signal of poor general hygiene and a high potential for pathogen presence. Indicators of fecal pollution were particularly widespread. Streptococcus spp., a primary marker for contamination from human or animal waste, was detected in nine of the ten OHD wells; three of these sites recorded counts greater than 100 cfu/100 mL, and two others had counts around 80 cfu/100 mL. Similarly, sulfite-reducing anaerobes (SRA), which signal more recent fecal contamination, were present in six wells, with two sites exceeding 100 cfu/100 mL. Opportunistic pathogen Pseudomonas spp. was found in seven wells with counts ranging up to 85 cfu/100 mL, indicating additional risks from environmental contamination. However, we note that the most severe targeted pathogens (including Salmonella, Shigella, and Staphylococcus) were not detected in any of our samples. The high concentrations of fecal indicator bacteria represent a major health concern, as WHO guidelines mandate their complete absence in safe drinking water. These findings confirm that shallow, unprotected wells, while potentially lower in chemical contaminants, pose a severe microbial risk. This illustrates the dangerous trade-off that communities in Turkana must navigate daily.

Fig 8: Percentage of elements of health significance per water site that were measured and are above the WHO and KS guideline thresholds

DISCUSSION

The results of this study demonstrate that potable groundwater sources in Turkana County, Kenya, are characterized by contrasting geogenic and microbial risks associated with different source types. Water quality varies markedly across the landscape, and no source category can be considered uniformly safe. Deep boreholes (DBH Solar), although hydrologically reliable, are characterized by elevated salinity and widespread excess of fluoride, sodium, aluminum, calcium, and iron, resulting in high salinity and chronic exposure risks. In contrast, shallow open hand-dug (OHD) wells in dry riverbeds often display comparatively lower mineralization but are highly vulnerable to fecal and microbial contamination. Communities therefore face a trade-off between chronic chemical exposure and acute microbial risk.

### General water chemistry

Elevated salinity was predominantly a result of sodium magnesium, iron, aluminum and calcium salts with concentrations of sodium, aluminum and iron exceeding both WHO and KS standards for drinking water in at least one-third of all the water sources sampled. Calcium was higher in the OHD wells and SBH pump as compared to DBH solar and springs that recorded a higher Na and Mg this is likely due to carbonate dissolution from upstream lithologies that is enhanced by Carbon dioxide rich infiltrating water.  Although calcium is an essential nutrient, excessive intake may contribute to water hardness, scaling in distribution systems, and for human health it can lead to potential kidney stone formation (WHO, 2017).

The exceedance of sodium taste threshold on over 24 % of all water sources and in 30% of all the samples from the SBH pump and 26% of all the DBH Solar illustrates the gravity of elevated salinity and salt burden on drinking water sources within the county. This is particularly significant considering that the median concentration in the OHD wells is a fraction of the concentrations on the deeper boreholes with a median of 58 mg/L compared to about 100 mg/L in the deeper water sources, which are the dominant watering points at the moment. Other important elements that exceeded set thresholds are Iron in 42% of all the water samples with about one-third of OHD wells exceeding set limits (> 0.3 mg/L), aluminum in 41% of all the water samples, Manganese in 8% of all the water samples and magnesium in at least 7% of all the water samples. OHD wells had particularly high concentrations of aluminum, iron and manganese but overall, with less salinity. While Iron is essential for oxygen transport and while manganese functions as a cofactor in numerous enzymatic processes, chronic overexposure, especially to manganese, has been linked to neurotoxicity (WHO, 2011).

Fig 9: Concentration of compounds of health significance, Boxplot dotted line represents WHO and KS guideline thresholds

These elevated levels are consistent with cation-exchange processes and volcanic lithologies, as confirmed by strong correlations (r > 0.6) between these ions, EC, and sulfate (SO₄²⁻). Elevated electrical conductivity (EC), total dissolved solids (TDS), and salinity in DBH Solar, particularly near Lake Turkana and other low-lying areas, reflect high mineralization driven by evaporative concentration, ion exchange, and prolonged rock–water interaction in these deeper relatively older aquifers. In contrast, shallow alluvial aquifers accessed through OHD wells display relatively low dissolved load (median EC ≈ 680 µS cm⁻¹; TDS ≈ 375 mg/L) and near-neutral pH (~7.7), suggesting rapid recharge and short residence times. Alkaline pH values (up to 9.2) in Kerio boreholes, without corresponding carbonate hardness, point to a geogenic activity or possible intrusion from lake-water. Boreholes in Lapur, Katilu, and Lokichogio recorded the highest CaCO₃ concentrations up to 759 mg/L, consistent with carbonate buffering or gypsum dissolution. Pairwise Wilcoxon rank-sum tests confirmed significant differences (p < 0.05) in EC, TDS, and CaCO₃ among source types. Excess salinity poses chronic health concerns: sodium intake is linked to hypertension and cardiovascular stress (Rosinger et al., 2021, WHO, 2012), while high magnesium may exacerbate gastrointestinal distress and dehydration, particularly among pregnant women and children. Potassium concentrations were locally elevated in the Lokichar cluster but generally remained within safe limits.

Fluoride showed widespread exceedance across all source types, with up to 40% of sampled watering points surpassing guideline values, recording a maximum of 26 mg/L measured at Lokamarinyang Spring against a WHO guideline (1.5 mg/L). DBH solar systems and springs recorded the highest median fluoride concentrations. A principal components analysis (PCA) showed that fluoride enrichment co-occurs with Na-HCO₃ facies, characteristic of fluorine-bearing volcanic terrains likely driven by prolonged water-rock interaction and high pH conditions that enhance fluoride mobility and that has been observed in other rift valley studies (Rusiniak et al., 2021). Chronic exposure to observed concentrations presents substantial risk of dental and skeletal fluorosis, a substantial, yet often under-recognized, public health burden across the East African Rift Valley. In high-fluoride regions of Kenya, prevalence rates of dental fluorosis have exceeded 80%, underscoring the scale of the problem and the urgent need for preventive interventions. (Kahama et al. 1997).

Trace and geogenic contaminants

Lead and arsenic are of equal health concern was recorded in isolated SBH pump watering points. Arsenic exceeded the WHO and KS limit (10 µg/L) in at least 4 water sources, 2 deep borehole locations in Kalokol and Lokiriama wards, SBH location in Lake zone ward and Lokamarinyang spring (Figure 6).  Elevate arsenic concentrations were observed at sites that had corresponding high pH values, this is likely leading to adsorption of arsenic from iron oxides due to reduced surface charge attraction. Lead concentrations were on average within both WHO and KS thresholds (10 µg/L), however,  SBH pump had on average higher median concentration (0.9 µg/L) as compared to the rest of the water sources that were all below 0.7 µg/L. Elevated concentrations occurred in aging hand-pump installations, suggesting infrastructure corrosion rather than a relation to aquifer chemistry.

Elevated vanadium concentration in a number of watering points indicated potential mobilization from mafic volcanic rocks. With a median concentration of 21.4 µg/L across all water sources and maxima if 500 µg/L being recorded. Although the World Health Organization has not established a formal guideline for vanadium in drinking water, several organizations use screening values around 15 µg/L – 21 µg/L as a precautionary benchmark. Given that a number of sources in Turkana County exceed these levels, chronic ingestion of such concentrations could potentially compromise the body with accumulation in bones and kidneys, leading to renal dysfunction (Clement, 1992).

Another set of elements that play either beneficial or detrimental roles depending on their concentration, chemical form, and exposure duration include zinc, copper, barium and phosphorus. While a number of them are largely recognized for their biological importance in bone metabolism, enzyme activation, and antioxidant defense they can be detrimental at elevated concentrations. Concentrations of zinc, copper and phosphorus largely remained with acceptable thresholds. Majority of these elements are associated with sulfates, often found anoxic conditions in sedimentary environments or with hydrothermal activities. While barium is common in evaporitic sequences with sulfate.

Boron and strontium are trace elements known for bone health and metabolism. However, elevated levels can affect developmental and reproductive health. Though not on the WHO watch list, Boron concentrations above 2 mg/L have been flagged as being potentially lethal especially for pregnant women and children. On average majority of the water sources were below this threshold with a median concentration of 0.17 mg/L across all water sources. However, at 6 watering points (1 spring, 1 SBH pump and 4 DBH solar) this threshold was exceeded. Concentrations of strontium were generally low and below concentrations that warrant health concern at a median of 0.5 mg/L. The negative co-variation of strontium and calcium suggests active carbonate precipitation along the flow path in majority of the water sources.

Uranium concentration exceeded the 30 µg/L threshold set by both the WHO and KS standards in at least 6 watering points (1 SBH pump and 5 DBH solar). Concentrations otherwise remained relatively low and below the threshold. At these concentrations, the dominant health concern is chemical nephrotoxicity (proximal tubular effects) from chronic ingestion rather than radiological dose (WHO, 2011).

### Microbial contamination and surface connectivity

Recent studies have profiled the Turkana communities as highly resilient. The resilience has mainly been shaped by their environment and traditional pastoralist diet (Lea et al., 2025). However, as majority of them transition to sedentary life the high microbial loads may lead to emergence of potential lethal microbiological assays as opposed to the fecal biota that their immune system may be used to. In addition, the prevalence of microbial pathogens in OHD wells confirm that open wells, while generally low in salinity, pose microbial risk.

Regional context and comparative perspective

Turkana’s hydrochemical profile is consistent with patterns observed across the East African Rift. Nowicki et al., (2023) reported that 96 % of groundwater samples across Kenya and Ethiopia exceeded at least one WHO guideline; our findings confirm Turkana as among the most affected regions, with simultaneous excesses of major salts linked to elevated salinity, fluoride and other essential and non-essential elements that exceeded set thresholds. Recorded fluoride maxima concentrations (26 mg/L), though from a spring exceed prior national estimates by Malango et al., 2017.

### Study limitations

Several limitations temper interpretation. Sampling occurred during a single field campaign and may not capture seasonal variation, particularly in shallow aquifers subject to recharge. Security constraints restricted spatial coverage to accessible sites, and microbiological tests were limited to OHD wells due to preservation challenges. Measurements reflect dissolved fractions only (filtered using a 0.22 µm membrane filters), possibly underestimating total exposure from colloid-bound elements. Finally, the absence of temporal data precludes trend analysis, however the dataset provides a spatially extensive hydrochemical baseline currently not available for Turkana County.

### Public health and management implications

The widespread excess of fluoride, salinity, and trace metals documented here shows that much of Turkana’s “safe” groundwater is chemically unsafe. Deep volcanic aquifers, a key solution for drought resilience, pose chronic health risks from fluoride, vanadium, and uranium exposure, while OHD wells present acute microbial hazards (sometime combined with chemical risk).

Given the prohibitive cost of centralized treatment, feasible mitigation options include local scale defluorination (bone char or activated alumina) and solar disinfection for shallow sources. Infrastructure monitoring should target replacement of corroded hand pumps and reinforcement of borehole casings. Routine monitoring should follow a tiered approach: quarterly testing of EC, F⁻, NO₃⁻, Mn, Al, and microbial indicators; annual screening for trace metals (As, U, V, B). Coupling these efforts with isotopic and hydrogeological mapping would enable predictive identification of high-risk aquifers. These recommendations follow the WHO’s Water Safety Plan (WSP) framework (REF) [ maybe we should ask an expert to expend on that].

At the policy level, enforcement of Kenya’s Water Act and county-level integration of chemical quality data into the Water Resource Authority’s database are critical steps. Transparent, community-accessible reporting will empower local decision-making and reduce inadvertent exposure. More broadly, Turkana illustrates how climate adaptation through groundwater reliance can unintentionally generate new health risk unless chemical safety is integrated into resilience planning.

CONCLUSION

This study documents widespread chemical and microbial contamination across Turkana County's diverse water sources. This situation is particularly critical given intensifying droughts and growing dependence on groundwater. Addressing these challenges requires moving beyond the traditional microbial-focused approach to water safety toward integrated chemical and biological monitoring, coupled with source-specific interventions specific to Turkana's hydrogeology. The data presented here provide the evidence base needed to guide such efforts and protect public health in one of Kenya's most water-vulnerable regions.

ACKNOWLEDGEMENT

This study was supported by the High Meadows Environmental Institute (HMEI), Princeton University, through a research grant administered by J. Ayroles. Water Grand Challenges Programs” and one to Niespolo and A. Fuentes including support for field, lab and personnel costs. Fieldwork was conducted under a research permit issued by the National Commission for Science, Technology and Innovation (NACOSTI), Kenya (Permit No. 989515), under the principal investigator F. Omengo.

We are deeply grateful to the Turkana Basin Institute (TBI) for their substantial technical, logistical, and infrastructural support, which was critical to the successful execution of the field campaign. We extend our appreciation to the County Government of Turkana, in particular the Directorate of Research and Innovation, the County Commissioner’s Office, and all Deputy County Commissioners, who facilitated access and issued formal authorizations to engage local administrative leaders across the various sub-counties and wards visited.

Special thanks go to the Chief Officer, Ministry of Water, Turkana County, for facilitating access to borehole inventories and coordinating linkages with local water officers. We also acknowledge the collaboration of the County Health Office, County Directorate of Education and Research, and numerous ward-level officials, chiefs, assistant chiefs, and village elders, whose cooperation was invaluable.

We are especially indebted to our field team: B. Kimathi, J. Echwa, and Mr. F. Emojo, whose endurance, commitment, and professionalism enabled the completion of extended field sampling across remote and challenging terrain. Finally, we express our sincere gratitude to the communities of Turkana, whose hospitality, trust, and participation made this work possible. They allowed us access to their lands and water resources, and for sharing traditional knowledge of water resource extraction. We recognize that many additional individuals and institutions played supportive roles, and we thank them for their generosity and goodwill.

REFERENCES

- Calow, R. C., MacDonald, A. M., Nicol, A. L., & Robins, N. S. (2010). Ground water security and drought in Africa: linking availability, access, and demand. Groundwater, 48(2), 246-256

- Clement Associates. (1992). Toxicological Profile for Vanadium and Compounds (Vol. 91, No. 29). Agency for Toxic Substances and Disease Registry, US Public Health Service.

- Cronin, A. A., Breslin, N., Gibson, J., & Pedley, S. (2006). Monitoring source and domestic water quality in parallel with sanitary risk identification in Northern Mozambique to prioritise protection interventions. Journal of Water and Health, 4(3), 333-345.

- Cuthbert, M. O., Taylor, R. G., Favreau, G., Todd, M. C., Shamsudduha, M., Villholth, K. G., & Kukuric, N. (2019). Observed controls on resilience of groundwater to climate variability in sub-Saharan Africa. Nature, 572(7768), 230-234.

- Dunkley, P.N., Smith, M., Allen, D.J., & Darling, W.G. (1993). The Geothermal Activity and Geology of the Northern Sector of the Kenya Rift Valley. British Geological Survey.

- Feibel, C.S. (2011). “A geological history of the Turkana Basin.” Evolutionary Anthropology, 20(6), 206-216.

- Gachet, A., Kooi, H., & Renard, P. (2020). "Groundwater resources in the Turkana Basin." Hydrogeology Journal, 28(1), 67-82.

- Healy, A., Tijani, M., Gronwall, J., Eichholz, M., Villholth, K. G., Mwango, F., ... & Gicheruh, C. (2022). Urban groundwater in Africa: A dialogue for resilient towns and cities.

- Kahama, R. W., Kariuki, D. N., Kariuki, H. N., & Njenga, L. W. (1997). Fluorosis in children and sources of fluoride around Lake Elementaita region of Kenya. Fluoride, 30(1), 19-25.

- Key, R.M. (1987). Geology of the Marsabit area. Report No. 108. Geological Survey of Kenya.

- KURIA. “Groundwater Distribution and Aquifer Characteristics in Kenya.” Kenya : a natural outlook : geo-environmental resources and hazards / 16 (2013): 83–107. Web.

- Leakey, M.G., Spoor, F., & Brown, F.H. (2011). “Early Homo fossils from Ileret, east of Lake Turkana.” Nature, 488, 201–204.

- Lapworth, D. J., Nkhuwa, D. C. W., Okotto-Okotto, J., Pedley, S., Stuart, M. E., Tijani, M. N., & Wright, J. J. H. J. (2017). Urban groundwater quality in sub-Saharan Africa: current status and implications for water security and public health. Hydrogeology journal, 25(4), 1093-1116.

- McDougall, I., Brown, F.H., Vasconcelos, P.M., Cohen, B.E., Thiede, D.S., Buchanan, M.J., 2012. New single crystal 40 Ar/39 Ar ages improve time scale for deposition of the Omo Group, Omo–Turkana Basin, East Africa. JGS 169, 213–226. https://doi.org/10.1144/0016-76492010-188

- McDougall, I., & Brown, F.H. (2009). Geochronology of the Turkana Basin. Cambridge University Press.

- Morley, C.K., Wescott, W.A., & Harper, R.M. (1999). "Geology and Hydrocarbon Potential of the Lokichar Basin." AAPG Bulletin, 83(2), 368-388.

- Omenda, P.A. (2013). “The Geology and Geothermal Potential of the East African Rift System in Kenya.” Proceedings, UNU Geothermal Training Programme.

- RTI International. (2013). Hydrogeological Assessment of the Lotikipi Basin Aquifer System. UNESCO report.

- Tiercelin, J.J., & Lezzar, K.E. (2002). “A 300-million-year history of rift lakes in Central and East Africa.” In The East African Great Lakes: Limnology, Palaeolimnology and Biodiversity. Springer.

- UNESCO and Radar Technologies International. Groundwater Resources Investigation in Northern Kenya and Southern Turkana: Discovery of New Aquifers Using WATEX Technology. UNESCO, 2013.

- Walsh, J. (1966). Geology of the Lodwar area. Geological Survey of Kenya.